# HX-400 technical proposal — Humber quench-water duty

Prepared for the reliability and process engineering teams. Revision C, 20 January 2026.

# Scope

Supply of one HX-400 removable-bundle shell-and-tube exchanger for quench-water service, including thermal and mechanical design, fabrication, inspection, and delivery to site. Extraction rail is in the base scope rather than a line item.

## Duty basis

Sized to the end of run rather than to a fouled steady state, on the cleaning interval the site actually holds. See

the HX-400 product page for the full configuration range.

- Design pressure 42 barg, design temperature 285 °C.

- Tube side duplex 2205; shell side carbon steel with a 3 mm corrosion allowance.

- TEMA AES, single shell pass, two tube passes.

- Surface area 412 m² at the specified end-of-run condition.

## Split of responsibility

| Item | Scope | Owner |
| --- | --- | --- |
| Thermal design report | Northwind | M. Ellison |
| Bundle extraction rail | Northwind, base scope | Works |
| Foundation and grouting | Client | Site civils |
| Insulation and cladding | Client | Site |
| Wet commissioning | Joint | Service |

## Delivery

Twenty-four weeks from drawing approval, held on the duplex mill slot already reserved. The bundle ships separately and is fitted on site by the regional workshop.

Figure 1 — bundle partly withdrawn on the extraction rail.